A Carter Thermo-Quad Guide

--------------------------

Author:  Gary Lewallen  (aka. Vaanth)

Date:    24 February 1998

-----------------------------------------------------------------------------

Copyright 1998  Gary Lewallen

-----------------------------------------------------------------------------

Contents...

-----------

= Introduction

= History and Description

- General

- Identification

= Subsystems

= Port Diagram

= Choices and Adaptations

= Numeric Listing

- Thermo-Quad

- Rochester Quadrajet

= Modification of Subsystems

= References

= Acknowledgements

-----------------------------------------------------------------------------

Introduction...

---------------

The Carter Thermo-Quad is a four-barrel carburetor with a spreadbore

throttle bore configuration.  It was designed as an emissions capable

carburetor that retained or surpassed secondary throttle performance of

earlier Carter carburetors, while delivering superior primary fuel

economy.  The Thermo-Quad consists of three main sub-assemblies,

an aluminum fuel bowl cover, a phenolic resin main body, and an

aluminum throttle base assembly.  The Thermo-Quad derives its name from

the phenolic main body.  Due to the material of the body, the carburetor

bowl can stay 20 degrees cooler than an all metal carburetor in the same

environment.  The Thermo-Quad was used by Chrysler Corporation in cars

and trucks 1971-1984.  Aftermarket versions were also produced by Carter.

This guide is intended to provide information for identifying Thermo-Quad

carburetors and related items.  It provides a basic history and basic

descriptions of the carburetor and its subsystems.  It is not intended

as a full theory of operation manual nor a repair manual.  Service

documentation should be consulted for repair procedures and service

details.  The factory service manuals provide good service procedures

and theory for specific models.  Other sources may be consulted for

general repair and modification procedures as well as theory of operation.

See the list of references for sources.

This guide is written with carburetor 'swapping' in mind.  Thus, certain

items are discussed with modification consideration (ie. emission subsystem

disablement) with the intent for adapting a non-original carburetor and/or

enhancing performance.  The modification and adaptation of the carburetors

may present legal issues, so consider the interest of any appropriate

government(s).

-----------------------------------------------------------------------------

History and Description...

--------------------------

General:

- - - -

The Thermo-Quad (TQ) was initially released for competition in 1969.  Chrysler

introduced the TQ on the 1971 340.  The first series of TQs including the

71 340 version and the Competition Series TQs were air metered units

unlike like the 72 and later TQs which were solid (liquid) fuel metered.

The Competition Series (CS) were available in 850 cfm and 1000 cfm ratings.

CS units use a manual choke and have a minimal amount of external attachments

compared to OEM production units. The CS was discontinued in the mid-70s.

Carter released the 9000 series in the latter 70s as replacement carburetors

for Chrysler and GM Quadrajet applications.  The 9000 series was very

similar to the production Chrysler Thermo-Quads.

In 1972, the OEM Chrysler TQs changed to the solid fuel metering type.  The

TQ coverage was expanded to include the 400 engine.  By 1973, all Chrysler

4-bbl applications were TQs (except some 413 truck models which continued to

use a Holley carburetor).  As the years progressed, the TQ evolved to meet the

continually tightening emissions requirements.  The changes were numerous.

Many features were added or modified externally and internally. The late

70s contained many variations for the various geographic regions, the

various features included/excluded, and the range of applications and engines

produced.  Into the 80s, the TQ became more complex, but year to year major

variation lessened somewhat.  In 1973, TQs received a port on the base for

canister purge and a port on the main body to provide a venturi vacuum signal

for EGR applications.  1975 saw the introduction of the Idle Enrichment system,

Altitude Compensator on some models, and the  Throttle Position Solenoid for

the new catalytic converter equipped cars.

In 1976, Chrysler introduced Lean Burn ignition and the TQ was modified

to produce and run on a very lean air/fuel mixture.  An external idle stop

switch and throttle position tranducer were added.  In 1978, the TQ bowl

vent was modified with the addition of an electric Bowl Vent solenoid.  An

additional rear base port for the vent hose replaced the bowl vent port. The

fuel inlet moved to the rear center of the carburetor from the previous

rear side location.  Lean Burn became Electronic Spark Advance (ESA) in

1979.  The very lean mixture idea was abandoned, but the electronic control

of the ignition advance was retained.  Hidden mixture screws were a feature

starting with some 1980 model TQs.  1981 introduced a riveted cover for

the choke pull-off linkage to prevent tampering and the oxygen feedback

solenoid on some models.  Idle Enrichment and Altitude Compensator was not

used on feedback models.  82-84 did not change much more in a major way.

The canister purge was eliminated by 84 in some applications and a power brake

port was added to the rear base.

After 1984, Chrysler stopped using Thermo-Quads.  Instead, the Rochester

Quadrajet was used through 1989 on cars and until 1988 in  trucks when

Electronic Fuel Injection replaced them.  Carter continued to supply the

fuel pumps for the Quadrajet equipped vehicles.  Although Chrysler was the

primary manufacturer to use the TQ, International Harvester (IHC) used them

in the late 70s and Ford used them in 1974.

The Thermo-Quad was available with two primary throttle bore sizes, 1-3/8"

and 1-1/2".  Flow ratings (CFM) vary depending on the source, but

the TQs with the 1-3/8" bores are listed as 750-800 CFM and those with the

1-1/2" primary throttle bores are rated at 800-850 CFM.  All TQs have the

2-1/4" diameter secondary throttle plates. The primary bore size depended

on application.  In general, all 78 and later 318s and 360s and all 340s had

the smaller bore.  Earlier 360s varied depending on application, most 400s

and all 440s had the large bore.  The 9000 series have the small primary bore

and were rated by Carter at 800 cfm.  Later TQs (ie, Lean Burn and ESA

applications, feedback systems) are quoted with less flow ratings, but

this is due to the control of the carburetion system, not the inherent

flow capability of the basic carburetor.  The internal metering is set for

leaner running conditions for Lean Burn.  Either bore size can be tuned

to run well on most engine combinations.  The smaller bore offers a slight

increase throttle response but less overall flow. The different bore sizing,

ie. speadbore, is an aspect that can lead to increased fuel economy while

delivering similar wide open throttle (WOT) performance to an equivalent

standard bore configuration.  The adjustability of the TQ and the

spreadbore configuration allows the use of a large CFM carburetor on

a small displacement engine.

The TQ gets its name from the phenolic resin main body.  This is "sand-

wiched" between the aluminum bowl cover assembly and the lower throttle

flange assembly.  The plastic body keeps the fuel ~20 degrees cooler

than an all metal carb in the same enviroment, leading to less percolation

problems and increased performance due to a denser charge.  The TQ is a

metering rod based carburetor like other Carters (AFB, AVS, BBD).  The

primary jets are housed in the plastic body, the primary metering rods

are suspended from the cover mounted step-up piston assembly into the

jets (in the 71 TQ and the CS, the primary jets are also housed in the cover).

Engine vaccuum (related to load) controls the position of the metering

rod in the jet, metering the fuel flow.  The secondary jets are suspended

from the cover.  Secondary flow is controlled by variable venturi effect in the

secondary bores.  The TQ secondary throttle plates are controlled by direct

mechanical linkage, air flow is controlled by a secondary, spring tension

resisted, air valve.  The valve is further controlled and damped by the choke

pull-off diaphragm.

The OEM TQs have a divorced choke (73-up with electric assist in most

applications), the 9000 series have an integral electric choke, and the

CS were equipped with a manual choke.  All OEM TQs and later CS have

screw-in jets.  Early CS retained the jets via O-rings.

Identification:

- - - - - - - -

To identify TQs, the model number is stamped on the lower left rear

bolt flange.  Earlier TQs also had a tag under one of the front bowl cover

screws, later ones  have a bar code sticker on the bowl cover with the

Carter model and/or a Chrysler part number.  International Harvester

TQs also have a tag under a front bowl cover screw.

Additional numbers will be found on the carburetor sections.  These

numbers are not used for TQ identification, but some can be used to relate

one casting to the other (note that parts with the same casting number

may be machined differently).  The upper bowl cover has the casting number

on the top, right of center rear: 6-XXXX (example: 6-2141, 6-2080, 6-2024).

The bowl has the casting number molded on the bottom of the right bowl

near the front, it is difficult to see with the throttle base on the carb:

0-XXXX (example: 0-2511A, 0-1823, 0-2709A).  The throttle base has the

casting number on the right upper side in a small recess: 1-XXXX (example:

1-2357, 1-2294, 1-2967.  More numbers will be found ink stamped, cast or

stamped in various areas.  Moreover, numbers are usually stamped below

the model number on the lower left rear bolt flange.  The model number

consists of four digits, usually followed by an 'S' (ie. 6318S).  The

model number is the primary and documented identifier.

The 71 OEM units can be identified externally by the mixture screws which

protrude at an angle from the base, in the same plane as the base, later

units protrude perpendicular to the base plane, but angle upwards from it.

Note: some 1980 and later units have hidden mixture screws.  The

Competition Series have raised pads on the upper bowl cover for a label.

Also, the CS have minimal external attachments, such as the lack of a PCV

port.  The mixture screws are similar to the 71 OEM TQs.

The fuel inlet on the CS and the 78-84 OEM units was located in the rear

center and the 71-77 OEM and 9000 series have the fuel inlet on the right

rear side.  The aftermarket 9000 series were available in 4 models:

9801, 9811, 9800, 9810.  The 9801, 9811 have Chrysler linkage (9801/9811

is EGR capable).  Note:  later OEM TQs model number started with a 9 as well,

but are not to be confused with the aftermarket 9000 series.

TQs with 1-3/8" primary throttle bore will have '2-315' stamped on the

throttle plates.  1-1/2" units will have '2-314' stamped on the plates.

All secondary plates are stamped with '2-312'.

-----------------------------------------------------------------------------

Subsystems...

-------------

This section briefly describes the primary Thermo-Quad subsystems that

are readily accessible and the various attachments for emissions.  For

detailed theory of operation of the subsystems or general operating

functions such as the low and high speed metering circuits, refer to

the references listed in the References section, specifically the

Carter Thermo-Quad service manual.  See the Service Parts Information

section for details on parts and part numbers.

- Jets:  jets are contained in the primaarry and secondary circuits, one

for each barrel.  Early CS units had press-in jets retained by o-rings.  CS

and 71 TQs have the primary jets in the upper bowl cover.  All other

TQs have screw-in primary jets in the main body.  All TQs have the

secondary jets mounted in the upper bowl cover.  All jets have a part number

prefix of 120-.  72 and later TQs usually have the part suffix stamped

on the jet, primaries: 4XXX, secondaries: 5XXX (or 120-5XXX).  The XXX

denotes the size, example:  4098 = 0.098", 5137 = 0.137".  CS and 71 TQs

have part number suffixes of 3XX or 3XXX.

- Metering rods and step-up:  metering rroods meter the fuel through the

primary jets.  They are essentially controlled by engine vaccuum and a

mechanical link, step-up cam/lever, connected to the primary throttle shaft.

Many variations of metering rods were available through the years.  The

depth of pre-76 models could be adjusted to tune, ( primarily ), cruising

condition flow via a screw adjustment.  Some later models retained this

feature, but starting in 1980 may have the adjustment locked via a collar.

Metering rods have three steps for metering, economy, midrange, power.

They are usually stamped with a part number, 75-XXXX or XXXX where the

XXXX defines the step sizes.  The CS and 71 TQs have numbers 16-XXX.

- Floats:  TQs are a single fed, dual innllet, dual bowl carburetor.  One

bowl feeds each carb half, ie. one primary and one secondary.  The dual

bowl arrangement is contained in the phenolic main body.  Dual floats

and dual inlet valves are employed.  Early TQs used brass floats.

Later models (73-74 and later) use nitrophyl floats.  All OEM TQs are

equipped with needle & seat number 25-1086 (0.0935" orifice).

- Accelerator pump:  the accelerator pummpp is located on the left front and

feeds from the left fuel bowl.  The pump is activated by the left side

throttle bracket.  There is some stroke adjustability at the upper lever.

The fuel is transfered via a plastic tube internally to the squirter

which resides above the primary venturi.  Later models have two adjusment

holes instead of three, and perform a two stage pump that add additional

fuel as the secondaries open.  Three accelerator pump clusters (squirters)

were available.

- Secondary air valve:  TQ secondary thrroottle plates are mechanically

linked to the primary on the left side.  As the plates begin to open,

the secondary air valve senses the opening and begins to open to provide

air flow which starts fuel flow from the secondary jets.  The initial

opening and rate is determined by a counteracting tension spring inside

the cover.  The opening is additionally regulated and dampened by the

choke pull-off assembly.  The air valve is contoured and the movement

provides a variable venturi effect.  A secondary fixed baffle is mounted

below the air valve.  Total air valve movement is limited by a protruding

tab.  The counteracting spring tension is adjustable via a slot/lock screw

on the left side to tune the rate.  Carter designed a special tool to

facilitate this adjustment.  Some of the CS TQs used an adjustment and

lock screw arangement similar to the AVS.

- Choke:  OEM TQs use a divorced, manifoolld mounted choke.  Exhaust crossover

heat operates the thermostatic spring contained in the choke well.  Most

73-84 TQs were electrically assist heated.  The electric control is via

a small module mounted to the intake or right head.  This unit times the

assist based on temperature and time and receives power from the ignition

run circuit.  9000 series have an integral choke assembly identified by

the black, circular plastic thermostat housing.  This unit is electrically

controlled.  CS units have a manual choke.  Choke action is accomplished

via the choke plate in the top of the primary side.

- Mixture screws:  screws used through 119976 had a 20 degree taper at the

seat.  In 1977, this was changed to 12 degrees to reduce adjustment

sensitivity.  Some ~77-79 TQs also contained internal restrictors to

limit adjustment.  Plastic caps with tabs to limit adjustment were also

installed on several models.  Many 1980 and later TQs had the base

redesigned to enclose the mixture screws so they could be 'capped' via

plugs after factory setting.

- Choke pull-off:  All TQ models.  This  pperforms the vacuum kick

pull-off function for the choke at initial cold engine start.  It is

also used to regulate and dampen secondary air valve opening.  Mounted

on the right rear base under a mounting bolt and screw.  Connects to

the rear vacuum port, right, color code gold.  The CS has the diaphragm

for secondary air valve control only.

- Hot Idle Compensator:  Some models aree  equipped with a compensator to

allow exta air into the mixture during high heat conditions.  If equipped,

it is located on the bowl cover over the secondary air valve.  High

temperatures can create an over-rich idle condition, and this compensates

for it, by allow extra air flow when it opens.

- Idle Stop Solenoid (ISS):  71-76.  Useedd to set the idle higher than the

basic curb idle screw for emissions reduction and to allow further

closure of the throttle blades at engine shutdown to prevent 'deiseling'

or 'run-on'.  Mounts on a bracket retained by base mounting bolt and

screw on the right side for 72-76.  71 mounted to the left side of the

intake under the carb linkage.

- Exhaust Gas Recirculation (EGR) port:    73-84, not used in some 76~81.

Some located on the main phenolic body as a tap into the venturi for vacuum

signal.  Venturi port is connected to a vacuum amplifier if used.  Some

models use a ported signal via a base port on the right front.  Base

ports color coded black.  Venturi ports are brass.

- Evaporative Control System (ECS) port::   73-84.  Used to purge the ECS

charcoal canister of collected gas fumes from the bowl vent and fuel

tank vent.  Color coded red.

- Idle Enrichment (IE) system:  75-81.   TThis is a driveability enhancement.

It supplements th choke function by allowing an even richer mixture during

the warm up period to improve driveability.  This will appear as a small

attachment on the front bowl cover that has a vacuum port that connects

to a coolant temperature sensor (CCIE), then to manifold vacuum with bleed.

It will also be plumbed into the EGR delay timing system.

- Altitude Compensator or Alcomp (Alc):    75-81.  This is another driveability

enhancement.  It appeared on most California and Federal high altitude

cars.  It appears as a small cylinder attaced to the front of the Idle

Enrichment system at the front of the bowl cover.  A small bellows inside

reacts to altitude changes and alters the air flow in the high speed

metering circuit.  This improves driveability and reduces emissions by

maintaining a correct fuel/air mixture.  Note:  later vehicles may be

equipped with a remote Alcomp sensor (fenderwell) to signal the feedback

solenoid controller to compensate.

- Bowl Vent (BV) solenoid:  78-84.  In aann effort to completely capture all

fuel bowl evaporative emissions, the standard bowl vent was redesigned

with a two way valve and holding solenoid at the rear of the carburetor.

When the engine starts, manifold vacuum pulls the valve rubber 'puck'

down, opening the bowl to the canister port.  The solenoid is connected

to the ignition run circuit and holds the valve open during low vaccuum

periods while running.  When the engine is shutdown, the valve releases and

reseals the float bowl.  Connects to rear base port, color code gold.

- Ground Switch:  76-84.  This appeared  wwith Lean Burn (LB).  It signals the

computer that the throttle plates are at idle position.  Later models

with solenoid idle stop combined the functions.  It is located on the

bracket on the right front.  A throttle shaft attachment contacts it.

- Throttle Position Transducer (TPT):  7766-81.  This appeared with Lean Burn.

It signals the position and movement rate of the throttle to the computer.

It is attached via the same bracket as the Ground Switch.

- Solenoid Idle Stop (SIS):  81-84.  Useedd to set the idle higher than the

basic curb idle when additional heavy accessory load (ie. rear window

defogger, air conditioning) is placed on the engine.  Mounts on a

bracket on the right front.

- Dashpot:  Some models, primarily truckkss, may have a dashpot to slow

throttle closing rate to reduce stalling.  Mounted on a bracket on the

left front.

- Vacuum Pull-Off Choke:  A few models mmaay have an additional pull-off

mounted on the left rear.  This connects to manifold vacuum via a control

switch in the vacuum plumbing.  It prevents choke operation after

engine warmup.

- Throttle Position Solenoid (TPS):  75--llater, some models.  Mounts in the

same place as the idle solenoids and is used to delay full throttle closure

at deceleration to prevent a momentary rich condition thus protecting

the catalytic converter(s).

- Vacuum Throttle Positioner (VTP):  75--llater, some models.  Mounts in the

same place and performs the same funcion as the Throttle Position Solenoid.

Also serves as a speed sensor.

- Pulse Solenoid, Oxygen Feedback Controoll (O2):  81-84.  Mounts on the

front of the bowl cover (where the IE module mounted earlier).  This

solenoid is used to control the air/fuel mixture via varying duty cycle

pulsing from the control computer based on feedback from exhaust, engine,

and ambient sensors.

- Fuel bowl inserts:  some later models  hhad an insert in the fuel bowls to

reduce the bowl fuel volume.

- Choke pull-off cover:  many 81-84 modeells have a cover to prevent field

adjustment of the pull-off.

-----------------------------------------------------------------------------

Port Diagram

------------

This section contains a diagram and notes to identify the various hose

port connections on the Thermo-Quad.  Different port configurations were

used throughout the years.  The diagram is a representation of the

throttle base, top view.  It is drawn with all possible ports.  The

label notes identify the ports and their general usage.  Some ports are

contained in the bowl or bowl cover.  These are also identified.  The ports

are labeled by (x)...descriptions follow the diagram.

FRONT

-----

___     _

(B)  |   |   | |

||  ||   |   |   | |    ||   //(F)

||  ||   |   |   | |    ||  //

-------||-------|---|---|-|----||------

| O	(A)	  (C)*	 (D)	(E)   O	|     /|

|					|    / |(G)

|					|   /  |

|	  -----          -----		|---  /

|	 /     \        /     \ 	|    /

|	|       |      |       |	|---

=|	|=O===O=|      |=O===O=|	|=

|	|       |      |       |	|

|	 \     /        \     / 	|

|	  -----          -----		|

|      -----------     -----------      |

|     /           \   /           \	|

|    |             | |             |    |

|    |             | |             |    |

=|    |===O=====O===| |===O=====O===|    |

|    |             | |             |    |

|    |             | |             |    |

|     \           /   \           /	|

|      -----------     -----------	|

|					|

|					|

|					|

| O (H)	 (I)  (J)  (K)    (L)	      O	|

---||--|---|-||--|---|---||------------   -------

||  |   | ||  |   |   ||               \     /

||  |   | ||  |   |   ||                -----

/   /     |   |                      //

/   /      |   |                       (M)

/   /       |   |

---         |   |

---

REAR

----

- (A) Distributor Vaccuum Advance (blackk,, 71-75, some 76-84), 5/32".

- (B) Idle Enrichment (IE) (gold, 75-81  ssome models), 5/32".

- (C) PCV (gold, 71-84), 11/32". * Note::  angled on 80-84 models.

- (D) Canister Purge (ECS) (red, 73-82),,  1/4".

- (E) EGR (gold) or Air Pump (blue), 5/3322".

- (F) EGR venturi port (brass), 1/8", {lloocated on phenolic float bowl}.

- (G) Bowl Vent (gold, 71-77), 11/32", ((llocated on the upper bowl cover}.

- (H) Air Cleaner Heated Air Temp Sensorr  (black, 78-84), 5/32".

- (I) Power Brake Tube (gold, 81-84), 111//32".

- (J) Air Cleaner Heated Air Temp Sensorr  (black 71-77), 5/32",

Bowl Vent Solenoid Vacuum Tube (gold 78-84), 5/32".

- (K) Canister Bowl Vent Tube (gold, 78--884), 11/32".

- (L) Choke Pull-off (gold, 71-84), 5/322"".

- (M) Choke Pull-off Diaphragm (natural,,  71-84), 5/32", {bolts to TQ base}

- some other vaccuum attachements were uussed, ie. secondary pull-off and

vacuum throttle positioner...these (like item M) were external to the TQ.

-----------------------------------------------------------------------------

Choices and Adaptations:

------------------------

Choices:

- - - -

Chrysler used the TQ from 1971-1984.  Ford used it in 1974 on some 460s

and International Harvester used it in 74, 75, 79, 80 on 345/392 engines.

The CS series was available from 69-~73.  The aftermarket 9000 series was

available from ~76-~late 80s.  The best OEM years to locate are 72-75.

TQs from this period have the least emission control add-ons.  The 71

OEM TQ performs well and has minimal emission considerations.  Due to

its air metered design, it is unlike the later TQs and few parts are

still available for it.  The CS TQ was not intended for street use, thus

it lacks provisions for many street engine items.  Carter released the

9000 series in the latter 70s.  It is a good unit for most applications.

It is the same design as the OEM TQ with minimal emissions devices.

In 76, Chrysler introduced Lean Burn (ELB) which evolved into Electronic

Spark Advance (ESA).  The 76-80 carburetors can be adapted for use in

earlier vehicles.  Many will need idle screws and vacuum ports added.

Most 78 and later TQs use a more complex bowl venting arrangement.  The

IHC carburetors are fairly simple like pre-76 Chrysler TQs.  1980 TQs began

began receiving a pulsing solenoid as part of an oxygen sensor feedback

system to allow more computer control of the mixture.  The TQs from this

period are the most complex and least desirable units.

There were several internal metering and passage changes that occured over

the years for emissions.  Most TQs can be tuned to match most applications.

The later carbs, however, will need extra adaptation for earlier vehicles.

The Competition Series carbs should generally be avoided except for racing.

The aftermarket 9000 series were  available in 4 models:  9801, 9811, 9800,

and 9810.  The 9801, 9811 have Chrysler linkage (9811 is EGR capable).

Note:  later OEM TQs model number started with 9 as well, not to be

confused with the 9000 series. The 9801/9811 is the prefered aftermarket TQ.

Because TQs are out of production (since 1985), new parts are scarce

except for common service items.  Kits are available through Carter,

Hygrade, KEM, etc.  Floats, choke pull-offs, choke assemblies are

also available.  Tuning parts, ie. jets, rods, are no longer available new.

Carter did make rod/jet kits ( Strip Kit ), but they are discontinued.

Because replacement jets and rods are no longer available, tuning can be

limited, unless a supply of used rods and jets is obtained.  Many

variations were used in the various applications over the years.  Jets and

rods can be modified.   The carburetors are easy to rebuild.  TQs usually

work well with just normal service adjustments.

Adaptations:

- - - - - -

Note:  for tuning basics and repair procedures see the References section

for possible information sources.  This section considers basic and

emission subsystem adaptation possibilities.

- TQ on 'squarebore' intake:  the TQ reqquuires a spreadbore intake manifold.

It can be adapted to a squarebore type via the use of an adaptor.  Several

companies make these (see Service Parts section).  Some performance can

be lost compared to a comparable spreadbore intake depending on the quality

of the adaptor and the transition area.  Vaccuum leaks can be an issue too.

- 1-3/8" vs. 1-1/2":  The TQs have two ddiifferent primary bore sizes.  These

units can be interchanged.  The smaller bore will yield slightly better

throttle response, but less overall flow.

- Jets/rods:  The jets and metering rodss  can be changed on the TQ to tune its

performance.  Many sizes were available.  See the Service Parts section for

sizing information.  Many later model TQs were lean on the primary side.

Increasing primary jet size and/or reducing metering rod size will richen

the primary side.  Secondary jets can be changed as well.  If replacement

units are not available, jets may be drilled to increase size.  Rods may

be filed to reduce size.  Solder and drilling/filing can be used to reverse

this to some extent.

- Floats:  Early TQs had brass floats.   BBy 1975, all TQs used nitrophyl

floats.  Over time, the nitrophyl floats can absorb gas and sink, allowing

fuel levels to be too high.  The floats should weigh 7.4-8.0 grams.  Carter

no longer services brass floats, but several sources are available (see

the Service Parts Information section).

- Accelerator pump clusters (squirters)::   The accelerator pump clusters were

available in various sizes.  Drilling may be used to increase the orifices

to tune pump fuel delivery.

- Mixture screws:  the screws are readillyy accessible on 71-79 TQs.  Some of

these had plastic caps to limit travel.  The caps can be removed.  Some ~77-79

contained internal restrictors to mixture adjustment.  These restrictions

are in the base mixture path from the idle circuits and could be opened if

needed.  Note:  mixture screws prior to 1976 had a 20 degree taper at the

seat and cannot be interchanged with 77 and later 12 degree screws.  1980

and later TQs may have hidden screws.  The base enclosed the screws in cast

protrusions under press-in caps.  The caps may be removed for mixture

adjustment.

- Interchanging bases, fuel bowls, upperr  bowl covers:  The three main TQ

sections are matched for each applications.  Most will interchange

mechanically, but flow and metering characteristics may not match.  The

sections have casting numbers that will allow some interchange identity,

but the casting machine work for applications may vary.

- Evaporative Control System (ECS):  73  aand later will have an extra base port

for the charcoal canister purge.  This is not a parasitic device and may

be retained without decreasing performance.  The bowl vent is plumbed to

the canister to vent fumes.  The purge pulls these fumes and those collected

from the tank.  The purge port can be capped to disable it.

- Exhaust Gas Recirculation (EGR):  EGR  ooccurs outside of the carburetor, but

tuning and vaccuum source provisions are incorporated into the various TQs

used on EGR equipped vehicles.  The tuning is usually not a major issue.

TQs may provide a venturi or a front base port for EGR vaccuum signals.  If

EGR is not used, these ports can be capped.

- Idle Enrichment system (IE):  this syssttem is a drivability enhancement.

It allows a richer mixture during the warm up period to improve driveability.

It supplements the choke function by blocking part of the air bleed to richen

it.  The TQ with IE will have a small attachment on the front bowl cover

that has a vacuum port that connects to a coolant temperature sensor (CCIE),

then to manifold vacuum.  It is also integrated into the EGR delay circuit.

The IE port can be retained or capped (if capped, the carb behaves normally).

- Altitude Compensator (Alcomp...Alc):   TThis device alters the high speed

metering circuit by sensing atmospheric pressure.  It mounts to the same

area as the IE system at the front of the bowl cover.  This system is also

a drivability enhancement, which also reduces emissions, by allowing the

TQ to compensate for elevation.  It could be disabled by blocking the

air passage ports (or the front vent pipe) to allow a normal full rich

condition for the high speed circuit.

- Solenoid Bowl Vent (BV):  Most 1978 anndd later TQs have an electric bowl

vent solenoid that replaces the earlier mechanical vent.  72-77 TQs used

a linkage actuated bowl mounted vent that fed to the ECS charcoal canister.

71 TQs actuated the same way, but vented to the oil breather or the atmosphere.

The later units vented through the rear base.  The vent mechanism closes

to vent to the ECS canister with the engine off.  When started, engine

vacuum opens the vent and the electric solenoid is powered to hold the

vent open during low vaccuum conditions.  The electric power is supplied

by the ignition run circuit.  While running, or off-idle for earlier TQs,

the bowls vent inside the air cleaner.  Vents should be retained for all

types.  The solenoid bowl vent could be redesigned to eliminate the

solenoid by adapting earlier parts, but the remachining effort is not

worth the result. The solenoid bowl vent system can be retained by wiring

into the ignition run circuit if adding to an earlier vehicle.  If the

ECS canister is not used the port can be vented to the atmosphere like

70 and earlier carburetors were done.  A protective screen over the port

can be used.

- Oxygen Feedback Pulsing Solenoid (O2)::    Most 1980 and later TQs will

contain an oxygen feedback controlled, pulsed solenoid in place of an IE

module.  The solenoid is variably pulsed by the control computer based on

the signal it receives from the exhaust mounted sensor.  The solenoid alters

the variable air bleeds by inputing more or less air based on computer

signal duty cycle.  If the solenoid is disconnected, the air bleeds will

allow a full rich condition.  Tuning with this in mind, this TQ could

potentially be used in a non-control/feedback vehicle.

- Idle Stop Solenoid (ISS):  the idle sttoop is external to the TQ and is used

to prevent engine run-on.  It can be retained, disconnected, or reused for

other functions such as compensating for engine loading (ie. air conditioning)

like a SIS unit.

- Solenoid Idle Stop (SIS):  the idle sttoop is external to the TQ and is used

to compensate for engine loading (ie. air conditioning).  It can be removed

if not needed.

- Throttle Position Solenoid (TPS):  thee  solenoid is external to the TQ and is

used for catalytic converter protection.  It can be removed if not needed.

- Vacuum Throttle Positioner (VTP):  thee  positioner is external to the TQ and

is used for catalytic converter protection and/or a speed sensor.  It can be

removed if not needed.

- Throttle Position Transducer (TPT):  hhee positioner is external to the TQ and

is used to signal throttle position and movement rate to the computer.  It

can be removed if not needed.

- Ground Switch:  the switch is externall  to the TQ and is used to signal

throttle closed to the computer.  It can be removed if not needed for a

computer or for idle adjustment.

- Idle screw adjustment on ELB and ESA TTQQs:

Lean Burn carburetor idle speed is controlled with the curb idle adjustment

screw that contacts the "throttle closed switch contact".  This contact

signals the Spark Control Computer that the engine is at idle, or not. The

contact is mounted on a bracket on the passenger side of the carburetor.

There is also a transducer, but it is to sense throttle position and

opening rate.  Some had the conventional idle screw based on application.

There are two approaches to the idle adjustment...

1) retain the "throttle closed switch contact" bracket (on the passenger side)

and adjust the idle there...this was the factory curb idle adjustment.

2) Drill and tap the boss for the conventional idle screw.  Drill the boss

perpendicular and near the center, but align the hole with the throttle

bracket extension.  The factory screw size is 10-32.  This approach yields

an idle adjustment screw like earlier TQs had.

- Adding vaccuum advance port to ELB andd  ESA TQs:

Many Lean Burn carburetors lack a vaccuum advance port because advance was

controlled by the ELB computer.  The port may be added to provide a

vaccuum advance signal for a conventional non-ELB/ESA distributor system.

The vacuum advance port is somewhat tricky and is best performed by comparison

to an earlier throttle base that has the factory port.  Using ~5/32"

drill bit, drill the throttle base where the vacuum port existed on

previous units....drill into the open (roughly square) opening between

the outside and the throttle bore.  This will accomodate the vacuum

fitting.  Then, using ~1/16" drill bit, drill two holes just above the

throttle plate into the same square opening from the throttle bore side

one hole adjacent to the other parallel to the throttle plate.  Then,

using a knife or small screwdriver, blend the holes together into a

slot.  This serves as the off-idle transfer slot to feed the vacuum

port.  Press a short lenght of 5/32" pipe or an old fitting into the

outside hole.  Vacuum advance is now available.  Using another base

with the port as a guide will ensure proper placement of the transfer

slot.  Misplacing the vacuum idle transfer slot can cause an off-idle

stumble and will necesitate additional tuning or replacement.

- Tamper-proof choke pull-off:  TQs withh  this feature have a pull-off

with the enclosure attached.  These can be replaced with an earlier

standard pull-off to allow choke tuning.

- Fuel bowl inserts:  these may be removveed to allow full bowl volume.

Other Considerations:

- - - - - - - - - - -

The TQ can possess the same problems as other carburetors.  Some of the

features of the TQ can create problems specific to its design.  Common

problems can usually be remedied with standard repair and tuning practices.

Some specific considerations are discussed in this section.

- Main jet wells:  The primary jets in 7722 and later TQs are screwed into a

threaded insert in the fuel bowl bottom.  This area is connected via a small

plastic channel to the internal fuel pickup area.  There is one channel for

each jet.  These channels are attached to the main body with an epoxy

adhesive.  The epoxy can weaken and begin leaking with age.  Correction can

be made by reattaching the wells with new epoxy (J-B Weld has been found to

work).  Remove the wells and the old epoxy.  Clean the area and reattach the

wells with a small bead of epoxy.  Remove any surplus epoxy that may

interfere with reassembly.

- Bowl warpage:  Due to the phenolic matteerial construction, the fuel bowl

can warp.  The area most prone to warpage is the upper edge corner of the

bowl at the rear.  Correction can be made by resurfacing the upper surface.

A large flat file can be used to resurface for corner warpage or other

irregularities.  Warpage of the bootom (unlikely) may require replacement

due to its configuration.

- Bowl breakage:  Severe breakage may reeqquire replacement of the fuel bowl.

Small breaks and cracks may be repaired with a fuel-resistant epoxy.  The

bottom surface has tabs for gasket location.  If these break, repair is not

required if the break does not extend into the bowl.  Small breaks around

the screw holes do not require repair if the break does not extend into

other areas.

- Idle transfer slot:  If the primary thhrrottle plates are opened too far in

an attempt to set the idle speed, the idle transfer slot may be exposed to

the air flow.  This can result in an off-idle hesitiation.  Correction can

be made by drilling a 1/16"-1/8" hole in the forward half of each primary

throttle plate.  The holes allow sufficient air flow for speed setting

without exposing the idle transfer slot.

- Step-up piston spring:  The step-up piisston spring resists vaccuum pull on

the piston.  In cases where idle vacuum is too low to allow stable holding of

the step-up piston, the spring may be trimmed to reduce its force.

- Commercial rebuilders:  Carburetors thhaat are mass rebuilt by commercial

rebuilders have their useful service life shortened or ended.  This usually

stems from mixing parts, bad service procedures, and cleaning by tumbling

or blasting.  Tumbling and blasting removes the factory finishes.  This leads

to accelerated wear and corrosion problems.  In the case of the TQ, the

factory linkage was cadmium or irridite finished.  The throttle shafts were

Teflon coated.  Tumbling or blasting removes these finishes and will lead

to corroded sticking linkages and binding or leaky shafts.  The TQ phenolic

fuel bowl can be damaged by tumbling or blasting.  Jets, rods and internal

orifices are further damaged by these processes.  Mixed parts without proper

analysis can result in a poorly performing or useless unit.  A carburetor from

a rebuilder of this nature requires careful examination to determine its future

usefulness.

- - - - - - - - - - - - - - - - - - - -  -- - - - - - - - - - - - - - - - - -

Numeric Listing...

------------------

Listing Notes:

- - - - - - -

This section attempts to identify most Thermo-Quads by the Carter model

number.  There were a multitude of models available with many configurations

to accomodate all the applications and varying regulations and conditions.

The listing identifies the models by primary application and year model.

There may be some variation in date availability and multiple application

models.  Most of this is reflected.  Abbreviations are used to highlight

specific features, particularly with California and Federal based emissions

and some major emission subsystems.

Known jet and metering rod information is included for the earlier years as

this was documented by Carter and Chrysler.  Later years were not documented

as well.  A kit reference number is given for each carb.  The kits are

referenced in the Service Information section.  The Carter 'Zip Kit' is

the primary reference.  Some conflicts arise in the documentation.  Carter

information is used where conflicts occur.

All primary and secondary jets are prefixed with 120- for TQs.  All metering

rods are prefixed with 75- for 72 and later OEM and 9000 series.  CS and 71

TQ rods are prefixed with 16-.

The prefixes are omitted from the listing to save space.

The 'Kit' field contains a code corresponding to a row code under the carb

kits in the Service Parts section.  The code is referenced to the Carter

rebuild kit or "Zip Kit".  Some competitor kit numbers have been provided.

These numbers have been cross-indexed to Carter's.  The cross-index is not

100% due to some manufacturer variances.  The most common cross-index has

been chosen in these cases.

Note that most TQ model numbers had an 'S' suffix.  Many will also appear

with a 'TQ' prefix.  These were left out of the list to save column space.

Where known, Chrysler part numbers are also listed in parenthesis.

Because the Rochester Quadrajet replaced the Thermo-Quad in 1985, a

brief listing of Quadrajets follows the TQs as reference.

Abbreviations:

- - - - - - -

AT = Automatic Transmission

MT = Manual Transmision   ( No reference means both transmissions )

Pri. = Primary Jet

Sec. = Secondary Jet

Rod = Primary metering rod

Bore = Primary throttle plate diameter in inches, All secondaries are 2-1/4"

HP = High Performance

CA = California application; generic for CA, N95

F = Federal application  ( No reference means both Federal and California.)

Can = Canada application. Typically, Canada used Federal carbs except noted.

LD = Light Duty

MD = Medium Duty

HD = Heavy Duty

LB = Lean Burn, ESA.  Noted on 76's.  After 76, most F cars were LB (N92, N94).

IE = Idle Enrichment

Alc = Altitude Compensator. Few noted, most CA and last years had Alc (N96).

HA = High Altitude

BV = Bowl Vent valve

O2 = Oxygen feedback solenoid

Thermo-Quad Listing:

- - - - - - - - - -

Model	Application			   Pri.	   Sec.	   Rod	   Bore	   Kit

------- ---------------------------	   -------   -----   ------  -----   ---

4846S	Competition Series		    331	    341	    615	   1-1/2    8

4846SA	Competition Series		   3080	   3116	    615	   1-1/2    8

4847S	Competition Series		    331	    341	    616	   1-1/2    8

4847SA	Competition Series		   3089	   3116	    616	   1-1/2    8

4972	71 340 MT	(3512820)	   3074	   3125	    655	   1-3/8    6

4973	71 340 AT	(3512821)	   3074	   3125	    655	   1-3/8    6

6090	72 400 AT F	(3614139)	   4095	   5137	   1937	   1-1/2    1

6138	72 340 MT F	(3614122)	   4095	   5137	   1940	   1-3/8    1

6139	72 340 AT F	(3614123)	   4095	   5137	   1940	   1-3/8    1

6140	72 400 MT F	(3614138)	   4095	   5137	   1939	   1-1/2    1

6144	72 340 AT Export		   4095	   5137	   1950	   1-3/8    1

6165	72 400 MT CA	(3614172)	   4095	   5137	   1938	   1-1/2    1

6166	72 400 AT CA	(3614173)	   4095	   5137	   1938	   1-1/2    1

6318	73 340 MT F	(3698327)	   4098	   5137	   1962	   1-3/8    1

6319	73 340 AT F	(3698328)	   4098	   5137	   1966	   1-3/8    1

6320	73 400 HP MT F	(3698332)	   4098	   5137	   1965	   1-1/2    1

6321	73 400 HP AT F	(3698333)	   4098	   5137	   1966	   1-1/2    1

6322	73 440 & Truck AT F (3698334)	   4098	   5125	   1966	   1-1/2    1

6324	73 440 HP AT F	(3698336)	   4098	   5125	   1966	   1-1/2    1

6339	73 340 MT CA	(3698339)	   4098	   5137	   1962	   1-1/2    1

6340	73 340 AT CA	(3698340)	   4098	   5137	   1966	   1-1/2    1

6341	73 400 MT CA	(3698342)	   4098	   5137	   1965	   1-3/8    1

6342	73 400 AT CA	(3698343)	   4098	   5137	   1966	   1-3/8    1

6394	72-73 340 AT Export					   1-3/8    1

6410	73 440  & Truck AT CA (3698319)	   4098	   5125	   1966	   1-1/2    1

6411	73 440 HP AT CA	(3698329)	   4098	   5125	   1966	   1-1/2    1

6446	M-440 Chrysler Marine	(3698397)			   1-1/2    1

6452	74 360 HP MT F	(3751433)	   4098	   5143	   1962	   1-1/2    1

6453	74 360 HP AT F	(3751434)	   4098	   5143	   2005	   1-1/2    1

6454	74 360 HP MT CA	(3751435)	   4098	   5143	   1962	   1-1/2    1

6455	74 360 HP AT CA	(3751436)	   4098	   5143	   2005	   1-1/2    1

6456	74 400 HP MT F	(3751439)	   4098	   5143	   1966	   1-1/2    1

6457	74 400 HP AT F	(3751440)	   4098	   5143	   1966	   1-1/2    1

6459	74 400 HP AT CA			   4098	   5143	   1966	   1-1/2    1

6460	74 440 AT F early (3751443)	   4098	   5137	   1966	   1-1/2    1

6461	74 440 AT CA early (3751444)	   4098	   5137	   2005	   1-1/2    1

6462	74 440 HP AT F early (3751445)				   1-1/2    1

6463	74 440 HP AT CA early (3751446)				   1-1/2    1

6488	74 360 AT CA early (3751451)	   4098	   5143	   1966	   1-1/2    1

6489	74 400 AT F early (3751452)				   1-1/2    1

6496	74 400 AT CA			   4098	   5143	   2005	   1-1/2    1

6503	74 400 AT Export					   1-1/2    1

6518	H-440-3 Chrysler Industrial      			   1-1/2    1

6545	74-77 440-1,-3 Truck F (3751472)   4098	   5137	   2024	   1-1/2    1

6550	74 345, 392 AT IHC (448582C91)				   1-3/8    1

6551	75 345, 392 AT IHC CA (459642C91)			   1-3/8    1

6568	74 460 AT Ford CA (D4AE-BC)				   1-3/8    7

6590	74 392 IHC (451872C91)					   1-3/8    1

6592	74 345, 392 MT IHC (451916C91)				   1-3/8    1

6614	74 ???				   4098	   5143	   1966	   1-3/8    -

6615	74 440-1 Truck AT		   4098	   5137	   2024	   1-1/2    1

6616	74 440-1 Truck AT CA		   4098	   5137	   1966	   1-1/2    1

9000	74 318 Export (3751474)					   1-3/8    1

9002	75-76 360 HP AT F IE  (3830529)	   4095	   5143	   1998	   1-1/2    2

9004	75 360 AT CA IE early (3830531)	   4095	   5125	   2086	   1-1/2    2

9008	75 400 AT CA IE early (3830535)	   4098	   5143	   1965	   1-1/2    2

9009	75 440 AT F IE    (3830536)	   4098	   5137	   2109	   1-1/2    2

9010	75 440 AT CA IE	  (3830537)	   4098	   5137	   1965	   1-1/2    2

9011	75 440 HP AT F IE (3830538)	   4099	   5137	   2103	   1-1/2    2

9012	75 440 HP AT CA IE early (3830539) 4098	   5143	   1965	   1-1/2    2

9013	74 360 AT CA late (3751420)	   4098	   5143	   1966	   1-3/8    1

9014	74 400 AT CA late (3751419)	   4098	   5143	   2005	   1-1/2    1

9015	74 440 HP AT late (3830415)	   4098	   5137	   2005	   1-1/2    1

9016	74 440 HP AT CA late (3830416) 	   4098	   5137	   1966	   1-1/2    1

9017	74 440-1 Truck AT CA (3830417)	   4098	   5137	   1966	   1-1/2    1

9019	72-74 ???						   1-3/8    1

9020	72-74 ???						   1-3/8    1

9022	74 360 Truck AT CA (3830401)	   4098	   5143	   1966	   1-3/8    1

9023	74 440 AT F late (3830403)	   4098	   5137	   1966	   1-1/2    1

9024	74 440 AT CA late (3830404)	   4098	   5137	   2005	   1-1/2    1

9025	74 440-1 Truck AT F (3830405)	   4098	   5137	   1966	   1-1/2    1

9027	75 392 IHC F (461280C91)				   1-3/8    1

9028	75 392 IHC CA (461281C91)				   1-3/8    1

9034	75 440-1 Truck MT F (3870944)	   4099	   5143	   1965	   1-1/2    1

9035	75 440-1 Truck MT CA (3870945)	   4100	   5125	   2110	   1-1/2    1

9036	75-76 440-1 Truck AT CA (3870946)  4100	   5125	   2110	   1-1/2    1

9046	75 400 AT F IE early   (3830554)   4099	   5143	   1965	   1-1/2    2

9047	76 360 AT F CA IE					   1-1/2    2

9049	74 440 HP AT CA late (3830864)	   4099	   5143	   1965	   1-1/2    2

9050	75 400 AT Can IE     (3830561)	   4099	   5143	   1965	   1-1/2    1

9051	75 440 AT Can IE      (3839562)	   4095	   5125	   2144	   1-1/2    2

9052	75-76 440 HP AT Can IE(3830568)	   4095	   5137	   2024	   1-1/2    2

9053	75 400 AT CA IE PDC-SW(3830570)	   4100	   5143	   1965	   1-1/2    2

9054	75-76 400 HP AT F IE  (3830569)	   4098	   5137	   2005	   1-1/2    2

9055	76 360 AT CA IE (4006624)	   4096	   5125	   1962	   1-3/8    2

9056	75 400 SW AT CA IE		   4099	   5143	   2145	   1-1/2    2

9057	76 400 AT Can IE		   4099	   5143	   1965	   1-1/2    2

9058	76 440 AT F IE (4006631)	   4099	   5137	   2109	   1-1/2    2

9059	76 440 AT CA IE	(4006632)	   4099	   5137	   2145	   1-1/2    2

9062	76 440 HP AT CA IE		   4098	   5137	   2153	   1-1/2    2

9063	75 440 AT CA early		   4099	   5143	   1965	   1-1/2    2

9064	76 400 AT LB			   4099	   5143	   1965	   1-1/2    2

9065	75-76 ???			   4098	   5137	   2109	   1-1/2    2

9066	76 440 HP AT F IE		   4099	   5137	   2103	   1-1/2    2

9068	75 400 AT CA late		   4099	   5143	   2145	   1-1/2    2

9069	75 440 AT CA late		   4099	   5143	   2145	   1-1/2    2

9072	75 360 AT CA late		   4099	   5143	   1965	   1-1/2    2

9073	75 440 AT CA late		   4098	   5137	   2145	   1-1/2    2

9074	76 400 AT CA (4006642)		   4099	   5143	   2145	   1-1/2    2

9076	77 360 HP AT CA IE (4027735)	   4092	   5137	   1938	   1-3/8    2

9077	77 400 HP AT F (4027736)	   4098	   5143	   2159	   1-1/2    2

9078	77 440 AT F IE (4027737)	   4098	   5137	   2159	   1-1/2    2

9080	77 440 HP AT F IE (4027739)	   4098	   5137	   2179	   1-1/2    2

9081	77 440 HP AT CA IE 		   4098	   5137	   1950	   1-1/2    2

9093	77 360 HP AT F CA IE (4027742)	   4092	   5143	   2195	   1-3/8    2

9094	76 440 HP AT F (4006648)	   4099	   5137	   2103	   1-1/2    2

9095	76 440 HP AT CA (4006649)	   4098	   5137	   2153	   1-1/2    2

9096	77 440 Truck AT CA (4041553)	   4099	   5125	   2110	   1-1/2    1

9097	76 400 AT LB (4006654)		   4099	   5143	   2086	   1-1/2    2

9099	76 400 AT CA late		   4099	   5143	   2145	   1-1/2    2

9100	76 400 AT Can late		   4099	   5143	   2144	   1-1/2    2

9101	77 440 AT CA IE (4027750)	   4098	   5137	   2210	   1-1/2    2

9102	77 400 HP AT F 						   1-1/2    2

9103	77 400 HP AT F (4027761)	   4098	   5143	   2211	   1-1/2    2

9104	78 360 AT F IE BV (4041826)	   4092	   5110	   1940	   1-3/8    3

9108	78 400 AT Can Exp (4041836)	   4098	   5143	   2159	   1-1/2    3

9109	78 440 AT F IE early (4041830)				   1-1/2    3

9110	78 440 AT F HA CA IE (4041831)				   1-1/2    3

9111	78 440 AT Can Exp IE (4041837)				   1-1/2    3

9112	78 440 HP AT IE (4041832)	   4098	   5125	   2179	   1-1/2    3

9114	77 440 AT Can			   4098	   5137	   2159	   1-1/2    2

9115	77 360 AT Alc (4027727)		   4092	   5137	   1938	   1-3/8    2

9116	78 440 Truck MD IE CA (4095406)	   4098	   5137	   1950	   1-1/2    3

9117	78 440 Truck HD CA (4095407)	   4099	   5125	   2110	   1-1/2    3

9118	78 440 Truck HD (4095423) 	   4098	   5125	   2110	   1-1/2    3

9119	77 440 HP AT CA (4027770)	   4098	   5125	   1950	   1-1/2    2

9123	78 318 Truck MT CA (4095427)	   4092	   5110	   1997	   1-3/8    3

9124	78 318 Truck AT CA (4095428)	   4092	   5110	   1997	   1-3/8    3

9125	78 360 Truck MT CA (4095429)	   4092	   5110	   1938	   1-3/8    3

9126	78 360 Truck AT CA (4095430)	   4092	   5110	   1938	   1-3/8    3

9127	77 440 AT			   4098	   5125	   2179	   1-1/2    2

9128	79-80 345 AT IHC  					   1-3/8    4

9134	78 360 AT F HA CA IE (4041856)	   4092	   5110	   1996	   1-3/8    3

9136	78 360 Mexico						   1-3/8    3

9137	78 318 AT Can Exp (4041858)	   4092	   5110	   1998	   1-3/8    3

9140	78 400 AT F  (4041861)		   4092	   5137	   1997	   1-1/2    3

9147	78 318 AT CA (4041865)		   4092	   5110	   1996	   1-3/8    3

9148	78 440 HP AT CA IE					   1-1/2    3

9149	78 440 Truck MD AT CA (4095440)				   1-1/2    3

9150	78 440 Truck HD AT CA (4095441)	   4099	   5125	   2110	   1-1/2    3

9151	78 440 Truck HD AT (4095442)	   4098	   5125	   1998	   1-1/2    3

9152	78 318 Truck AT LD Can		   4092	   5110	   1997	   1-3/8    3

9153	78 440 HP AT CA (4041874)				   1-1/2    3

9161	78 440 AT Can Exp (4041879)				   1-1/2    3

9172	79 360 Truck LD MD BV IE				   1-3/8    3

9173	78-79 360 Truck HD BV					   1-3/8    3

9180	78 440 AT F  (4041881)					   1-1/2    3

9182	78 400 AT F  (4041882)					   1-3/8    3

9183	78-79 318 Truck BV					   1-3/8    3

9185	78-79 318 Truck	BV					   1-3/8    3

9187	78-79 318 Truck	BV					   1-3/8    3

9188	78 400 AT Can Exp (4041892)				   1-3/8    3

9190	78 440 Truck MD AT CA					   1-1/2    3

9193	78-79 360 Truck HD BV					   1-3/8    3

9194	78-79 360 Truck	HD BV					   1-3/8    3

9195	79 318 AT CA IE BV (4095934)				   1-3/8    3

9196	79 360 AT F IE BV (4095935)	    4095   5110   2210?	   1-3/8    3

9197	79 360 AT Can IE BV (4095936)				   1-3/8    3

9198	79 360 HP AT CA IE BV (4095937)				   1-3/8    3

9202	79 360 AT CA IE BV (4095939)				   1-3/8    3

9203	79-80 345 AT IHC (1700004C91)	    4095   5110   2144	   1-3/8    4

9205	79-80 345 AT IHC 		                      	   1-3/8    3

9207	79 360 Truck HD						   1-3/8    3

9208	78-79 360 Truck HD BV					   1-3/8    3

9209	79 360 Truck HD						   1-3/8    3

9210	78-79 360 Truck HD BV					   1-3/8    3

9211	79 440 Truck HD						   1-1/2    3

9212	79 440 Truck HD AT CA					   1-1/2    3

9213	78 440 Truck HD						   1-1/2    3

9214	78-79 440 Truck BV					   1-1/2    3

9215	78-79 ??? Truck						   1-3/8    3

9216	78-79 318 Truck	BV					   1-3/8    3

9217	78-79 360 Truck HD BV					   1-3/8    3

9218	78-79 360 Truck	HD BV					   1-3/8    3

9219	78-83 ??? Truck						   1-3/8    3

9221	78 440 Truck HD AT					   1-1/2    3

9223	78-79 318 Truck BV					   1-3/8    3

9224	79 360 Truck LD MD BV IE				   1-3/8    3

9225	79 360 Truck LD MD BV IE				   1-3/8    3

9226	79 360 Truck LD MD BV IE				   1-3/8    3

9227	78-79 318 Truck BV					   1-3/8    3

9228	78-79 318 Truck BV					   1-3/8    3

9229	78-79 318 Truck BV					   1-3/8    3

9230	80 318 AT Can Exp BV (4179033)				   1-3/8    3

9232	80 318 AT Can BV (4179035)				   1-3/8    3

9234	80 318 AT F BV (4179037)				   1-3/8    3

9236	80 360 AT Can BV (4179039)				   1-3/8    3

9238	79-81 ??? Mexico             				   1-3/8    3

9240	79 446 Motor Home AT *					   1-1/2    3

9241	79 446 Motor Home AT CA	BV *				   1-1/2    3

9243	80 318 AT Can Exp BV (4179043)				   1-3/8    3

9244	80 360 AT F BV Alc (4179044)				   1-3/8    3

9245	79 318 AT CA BV (4095495)				   1-3/8    3

9246	79 360 AT CA BV IE (4095496)				   1-3/8    3

9247	79 440 Truck AT						   1-1/2    3

9248	79 440 Truck AT CA					   1-1/2    3

9250	79 360 AT CA BV (4095978)				   1-3/8    3

9251	80 360 Truck MD CA					   1-3/8    3

9252	80 360 Truck MD CA					   1-3/8    3

9254	80 318 Truck BV						   1-3/8    3

9255	80-85 360 Truck LD MD F					   1-3/8    3

9256	79 318 AT F Alc (4095979)				   1-3/8    3

9265	80 360 Truck LD MD F					   1-3/8    3

9266	80 360 Truck LD MD F					   1-3/8    3

9268	79 360 Truck LD MD BV IE 				   1-3/8    3

9269	78-79 318 Truck	BV					   1-3/8    3

9270	78-79 318 Truck	BV					   1-3/8    3

9271	78-79 318 Truck	BV					   1-3/8    3

9272	79 360 Truck LD MD BV IE   				   1-3/8    3

9273	79 360 Truck LD MD BV IE				   1-3/8    3

9275	78-79 360 Truck HD BV					   1-3/8    3

9276	78-79 360 Truck	HD BV					   1-3/8    3

9277	79 360 Truck HD						   1-3/8    3

9278	78 360 Truck HD						   1-3/8    3

9279	80 318 Truck						   1-3/8    3

9280	80-85 360 Truck	LD MD F					   1-3/8    3

9281	80 360 Truck HD						   1-3/8    3

9283	81 318 AT BV F (4179132)				   1-3/8    3

9284	81 318 AT BV Can Exp (4179133)				   1-3/8    3

9288	80 318 Truck LD Can					   1-3/8    3

9289	80 446 Motor Home AT BV *				   1-1/2    3

9291	80 446 Motor Home AT BV *				   1-1/2    3

9292	80 360 Truck						   1-3/8    3

9293	81 318 AT BV F	(4179145)				   1-3/8    3

9295	80 318 AT BV F Alc (4179051)				   1-3/8    3

9296	80 318 Truck MD HD					   1-3/8    3

9298	80 360 Truck LD MD F					   1-3/8    3

9299	80 360 Truck LD MD F					   1-3/8    3

9304	81-83 ??? Mexico             				   1-3/8    -

9305	80 360 AT F						   1-3/8    3

9306	80 318 O2 BV						   1-3/8    3

9311	81 318 Truck LD						   1-3/8    3

9314	81 360 Truck LD MD F					   1-3/8    3

9320	80 318 O2 BV						   1-3/8    3

9325	81 360 Truck LD Can					   1-3/8    3

9326	82 360 Truck LD MD F					   1-3/8    3

9327	83 360 Truck LD MD F					   1-3/8    3

9329	82 318 Truck LD						   1-3/8    3

9330	83 318 Truck LD						   1-3/8    3

9331	82 360 Truck LD MD CA Alc				   1-3/8    3

9332	83 360 Truck LD MD CA Alc				   1-3/8    3

9340	83-84 ??? Mexico             				   1-3/8    3

9341	83 318 Truck LD						   1-3/8    3

9342	82 318 Truck BV (4287013)				   1-3/8    3

9357	83 318 Truck MD HD					   1-3/8    3

9358	82 360 Truck HD						   1-3/8    3

9359	83 360 Truck HD						   1-3/8    3

9364	81-84 318 AT Can BV O2 (4179177)			   1-3/8    3

9365	83 318 Truck LD						   1-3/8    3

9366	84 360 Truck LD MD F					   1-3/8    3

9367	83 318 Truck LD						   1-3/8    3

9368	83 318 Truck LD						   1-3/8    3

9369	84 360 Truck LD MD CA Alc				   1-3/8    3

9370	84 360 Truck HD						   1-3/8    3

9371	84 318 Truck MD HD					   1-3/8    3

9372	81-82 318 AT F BV O2 (4179179)				   1-3/8    3

9373	81 318 AT CA BV O2 (4179180)				   1-3/8    3

9374	82-83 318 BV O2						   1-3/8    3

9375	82 318 Truck BV (4241752)	 			   1-3/8    3

9376	82 360 Truck BV CA (4241753)				   1-3/8    3

9379	83 360 Truck BV F (4287016)				   1-3/8    3

9380	79-84 ??? Mexico             				   1-3/8    -

9385	82-83 318 BV O2						   1-3/8    3

9386	84 360 Truck BV Alc					   1-3/8    3

9387	84 360 Truck BV						   1-3/8    3

9388	83 318 AT BV O2						   1-3/8    3

9389	84 318 AT BV O2 (4300012)				   1-3/8    3

9391	84 318 AT BV O2 HA (4300019)				   1-3/8    3

9800	9000 series, GM			4095	5137	2110	   1-3/8    5

9801	9000 series, Chrysler		4095	5125	2110	   1-3/8    5

9810	9000 series, GM, EGR		4095	5137	2110	   1-3/8    5

9811	9000 series, Chrysler, EGR	4097	5125	2385	   1-3/8    5

* Note:  The 446 engine used in 79-81 Dodge motorhomes was built by IHC.

- - - - - - - - - - - - - - - - - - - -  -- -

Rochester Quadrajet (M4ME & E4ME) Listing:

Note: O2 denotes the E4ME feedback unit.

Note: identification number is stamped vertically on the left side near

the secondary throttle lever.

17085407  85-87 318  O2

17085408  86 360 Truck AT F (4306408)

17085409  86 360 Truck AT F HA (4306409)

17085411  85 318 Can Exp

17085411  86-87 318 (4306411)

17085414  85-88 360 Truck HD

17085415  85 360 Truck HD

17085416  85-88 360 Truck

17085417  86 360 Truck MT F (4306417)

17085425  86 360 Truck F (4306425)

17085431  86 360 Truck HD CA Can (4306431)

17085432  85-88 360 Truck HD

17085433  85-89 318 O2 (4306433)

17085434  86 360 Truck F (4306434)

17087175  85-88 360 Truck

17087176  85-88 360 Truck

17087177  85-88 360 Truck

17087245  85-88 360 Truck HD

-------------------------------------------------------------------------------

Service Parts Information...

----------------------------

Metering Rod Info (dimensions in inches):

- - - - - - - - - - - - - - - - - - - -  --

16-XXX is for 71 OEM and CS TQs.  75-XXXX is for 72 OEM and 9000 series.

Step sizes listed, Econ = economy, Mid = midrange, Power = power steps.

Rod #	Econ	Mid	Power

-------	----	----	-----

16-615	.059	 --	.040	For CS only.

16-616	.064	 --	.030	For CS only.

16-632	.057	 --	.040	For CS only.

16-633	.055	 --	.040	For CS only.

16-634	.062	 --	.030	For CS only.

16-635	.060	 --	.030	For CS only.

16-655	.062	 --	.040	For 71 OEM TQ only.

16-683	.060	 --	.035	For 71 OEM TQ only.

16-683	.058	 --	.035	For 71 OEM TQ only.

75-1937	.070	.046	.040

75-1938	.071	.051	.040

75-1939	.070	.058	.040

75-1940	.072	.060	.045

75-1962	.067	.055	.045

75-1965	.069	.061	.045

75-1966	.067	.052	.045

75-1994	.068	.043	.035

75-1995	.066	.040	.035

75-1996	.069	.048	.035	*

75-1997	.067	.045	.035	*

75-1998	.070	.057	.040

75-1999	.068	.054	.040

75-2000	.066	.054	.035

75-2001	.064	.051	.035

75-2002	.065	.052	.040	*

75-2003	.063	.049	.040	*

75-2004	.067	.058	.040	*

75-2005	.065	.055	.040	*

75-2024	.066	.054	.040

75-2086	.069	.061	.040

75-2103	.069	 --	.045

75-2109	.069	.061	.045

75-2110	.069	.053	.040

75-2144	.069	.061	.050

75-2145	.069	.061	.055

75-2153	.067	 --	.055

75-2159	.070	.062	.040

75-2179	.074	.066	.042

75-2210	.069	 --	.045

75-2211	.0715	.064	.040

75-2385	.069	 --	.053

Primary Jet Info (dimensions in inches):

- - - - - - - - - - - - - - - - - - - -

120-3XXX is for 71 OEM and CS with screw-in jets.  120-3XX is for CS with

press-in jets.  120-4XXX is for 72 and later solid fuel OEM & 9000 series.

Jet #		Size

-------		----

120-330		.077

120-331		.080

120-332		.083

120-333		.086

120-334		.089

120-335		.092

120-3074	.074

120-3077	.077

120-3080	.080

120-3083	.083

120-3086	.086

120-3089	.089

120-3092	.092

120-4092	.092	*

120-4095	.095	*

120-4097	.097

120-4098	.098	*

120-4099	.099

120-4100	.100	*

120-4101	.101	*

120-4104	.104	*

* denotes part of 10-203 Strip kit.

Secondary Jet Info (dimensions in inches):

- - - - - - - - - - - - - - - - - - - -  --

120-3XXX is for 71 OEM and CS with screw-in jets.  120-3XX is for CS with

press-in jets.  120-5XXX is for 72 and later solid fuel OEM & 9000 series.

Jet #		Size

-------		----

120-339		.110

120-340		.113

120-341		.116

120-342		.119

120-343		.122

120-344		.125

120-3113	.113

120-3116	.116

120-3119	.119

120-3122	.122

120-3125	.125

120-3128	.128

120-3131	.131

120-5110	.110

120-5125	.125

120-5131	.131	*

120-5137	.137	*

120-5143	.143	*

120-5149	.149	*

* denotes part of 10-203 Strip kit.

Needle and Seat Info (dimensions in inches):

- - - - - - - - - - - - - - - - - - - -  -- -

Needle/Seat #	Size

-------------	------

25-1068		.111		For CS only.

25-1069		.120		For CS only.

25-1070		.125		For CS only.

25-1086		.0935	*	(stock OEM)

25-1091		.125

25-1103		.111	*

* denotes part of 10-203 Strip kit.

Accelerator Pump Cluster (squirter) Info (dimensions in inches):

- - - - - - - - - - - - - - - - - - - -  -- - - - - - - - - - - -

Cluster #	Size

---------	------

48-365s		.0

48-369s		.031  *

48-370s		.035  *

48-374s		.075  *

121-662	        gasket *

* denotes part of 48-380U package.

Carb Kits (Zip Kits):

- - - - - - - - - - -

Ref   Carter	Hygrade	  Kem	   B-W 	  Filko	    Echlin   Chrysler

---  ---------	-------	 ------	 -------    -------   -------  --------

1   902-299B	  657C	 15543A	 10416A	  24-2262A  15543A    3837521

2   902-310A	  657C	 15605	 10478	  24-2268   15605     4131180

3   902-329C	  1574   15711C	 10634C   24-2276   15711C    4271945

4   902-330A     1423   15711C   ---      ---      15711C      ---

5   902-318A	  ---*	  ---     ---      ---       ---  	---

6   902-292	  551     ---    10373     ---       ---      3621225

7   902-309	  676	 15554A	  ---	   ---	    15554A    CT-1062 **

8   5153x 	  ---     ---     ---      ---       ---        ---

* 902-318A is for the 9000 series, Hygrade 657C is known to work in a 9811.

** CT-1062 is the Motorcraft number for the Ford application.

Carter Strip Kit:

- - - - - - - - -

Carter 10-104 Strip Kit (Competition Series)

Carter 10-105 Strip Kit (Competition Series...SA)

Carter 10-110 Strip Kit (71 and Competition Series)

Carter 10-203 Strip Kit (72-up) (see Strip Kit section for early versions)

Floats:

- - - -

Carter 21-252 (nitrophyl), 21-249 (brass)

Chrysler 3780155

IHC 501288-C1

Ford D4AZ-9550-B, CM2028

KEM F104 (brass)

Borg Warner CF252B (brass), CF252 (nitrophyl)

Echlin Fuel Systems 2-483 (brass), 2-427 (nitrophyl)

FILKO  19-105 (brass)

Motorcraft CM-4384 (brass)

Hygrade FL4 (nitrophyl)

Chokes:

- - - -

Note:  Most of the Chrysler TQ chokes were of the same basic design.  In

1973, electric assist was added.  Following is a summary of most of the

chokes with model applications.  Various versions were available due to

operating environments and emissions.  The listing is per Carter choke

numbers from their catalog.  Some manufacturer cross-indexing differs.

Chrysler and Hygrade listed for reference....

Carter 170-148. Chrysler 3512875:

4972,4973,6138,6139,6144

Carter 170-245:

6090,6140,6165,6166

Carter 170-1371. Chrysler 4287004. Hygrade CV400 (non-electric assist):

9379 (Canada)

Carter 170-1408. Chrysler 3698355,3751484,4027715,4027793,4095328. Hygrade

CV164:

6318,6319,6320,6321,6322,6324,6339,6340,6341,6342,6410,6411,6456,6457,6459,

6460,6461,6462,6463,6503,9008,9009,9010,9011.9012,9014,9015,9016,9023,9024,

9046,9049,9050,9051,9052,9053,9054,9056,9057,9058,9059,9062,9063,9064,9065,

9066,9068,9069,9072,9073,9074,9077,9078,9080,9081,9094,9095,9097,9099,9100,

9101,9102,9103,9108,9112,9114,9116,9119,9127,9140,9149,9153,9182,9188,9190,

9221,9243,9251,9252,9254,9275,9279,9281,9288,9292,9296,9359,9370,9375,9376,

9379

Carter 170-1409 (non-electric assist):

6518,9000,9117,9118,9150,9151,9212,9214,9357,9358,9371

Carter 170-1490. Chrysler 4027792,4095984,3830548,4027714,4095341. Hygrade

CV172,CV174:

6452,6453,6454,6455,6488,9002,9004,9055,9076,9093,9104,9115,9123,9124,9125,

9126,9134,9136,9137,9147,9152,9173,9183,9185,9187,9193,9194,9195,9196,9197,

9198,9202,9207,9208,9209,9210,9211,9215,9216,9217,9218,9223,9224,9225,9226,

9227,9228,9229,9245,9246,9250,9255,9256,9265,9266,9268,9269,9270,9271,9272,

9273,9276,9277,9278,9298,9299,9311,9314,9329,9330,9331,9332

Carter 170-1598 (non-electric assist):

9234,9236,9244,9305,9306,9320

Carter 170-1621. Chrysler 4095331. Hygrade CV241:

9325,9341,9342,9364,9365,9366,9367,9368,9369,9386,9389

Carter 170-1675:

9283,9284,9293,9364,9372,9374,9385,9388

Carter 170P-1322A:

9800,9801,9810,9811  (integral choke, 9000 series)

Also note:

73-up without electric assist: Chrysler 4027795, Hygrade CV370.

Choke Pull-offs:

- - - - - - - -

Note:  Most of the TQs used the same choke pull-off.  Many 81-84 units

used the same diaphragm with a different bracket for the tamper-resistant

housing.  Some 81-84 Dodge trucks, IHC units and Ford used a secondary

pull-off.  The listed pull-offs cover most applications...

Carter 202-742, 202-664 (71-up)

Chrysler 3621210, 4049239, 4049240, 4049275, 4131250 (71-up)

Hygrade  CPA77 (71-up)

IHC 460369C1 (74-80)

Ford D4AZ-9J549a  (74)

KEM CP146 (71-up)

Carter 202-818 (81-84 tamper-resistant)

Chrysler 4267133 (81-84 tamper-resistant)

Hygrade CPA276   (81-84 tamper-resistant)

KEM CP319 (81-84 tamper-resistant)

Carter 202-821 (81-84 secondary pull-off)

Chrysler 4267137 (81-84 secondary pull-off)

Hygrade CPA269   (81-84 secondary pull-off)

KEM CP320 (81-84 secondary pull-off)

Carter 202-981 (84 secondary pull-off, vacuum contolled)

Miscellaneous Parts:

- - - - - - - - - -

O-ring, main well cavity: Carter 163-90, Chrysler 3685655 (or 3549406)

Needle & Seat:  Chrysler 3621161,  Carter 25-1086 (OEM size), Hygrade VN87C

Accelerator pump:  Chrysler 4094867, Carter 64-465

Spreadbore/squarebore adaptor:  MP P4007522, Edelbrock # 2691.

Air horn gasket:  Chrysler 53030600

1/4" spacer base gasket:  Chrysler 3698365

Set of four, 1/4" spacer base gasket (3698365):  MP P4007902

Solenoid Bowl Vent:  Carter:  213-93 (78), 213-145 (79-84 Truck, 79-80 CA),

213-211 (81 CA MT, Can, 81-84 Export), 213-111 (80-84

Mexico), 213-229 (CA AT Truck), 213-240 (81 CA late Truck)

Pulsing Solenoid (O2):  213-193 (81-84 318 car)

Tools:

- - -

Air Valve Tool:  Carter 109P-397, Miller Special Tools # C-4152B

- - - - - - - - - - - - - - - - - - - -  -- - - - - - - - - - - - - - - - - -

Strip Kits...

-------------

Carter produced Strip Kits in the 70s through the mid-80s for the Thermo-

Quad.  These are tuning kits for modifying the performance of the TQ.  They

consist of various metering rods, primary and secondary jets and needle/seats.

There were kits for the Competition Series and specific kits for the early

OEM/9000 TQs.  Later, Carter consolidated the kits for the 72-later OEM and

9000 series into a single kit.  Carter also offered kits for the WCFB, AFB,

and AVS four barrels.  A current version of the AFB kit is still available.

All the TQ kits and others have been discontinued.  Following is a listing

of the early and late TQ kit contents.  The other kits are also listed as a

reference.

Early Kits, CS and other carbs:

- - - - - - - - - - - - - - - -

Kit #	Carb Type and Models Covered

------	---------------------------------------------

10-101	AFB 3783, 4758, 4759, 4761, 9500, 9625

10-102 	AFB 4760, 4762

10-103	AFB 3361, 3720, 3721, 3804

10-104	TQ CS4846S, CS4847S (press-in jets)

10-105	TQ CS4846SA, CS4847SA (screw-in jets)

10-106	AVS all

10-107	AFB 4139, 4324, 4325, 4343, 4402, 4430, 4431, 4432, 4619, 4620, 4621,

4742, 4745, 4746, 4969, 4970, 4971

10-108	WCFB 2669, 2816, 2817, 2818, 3059, 3060, 3061, 3190, 3191, 3500,

3501, 3696, 3697

10-109	WCFB 2214, 2218, 2219, 2330, 2351, 2361, 2366, 2419, 2505, 2574,

2575, 2626, 2627, 2656, 2657, 2829, 2830, 3181, 3769

Early Kits, OEM and 9000 TQ:

- - - - - - - - - - - - - -

Kit #	Carb Type and Models Covered, with contents

------	---------------------------------------------

10-110	TQ 4972, 4973 ('71 340)

- primaries:  .074, .077, .080, .083

- secondaries:  .119, .122, .125, .128, .131

- needle/seats:  .0935", .111"

- rods:  655, 682, 683

10-111	TQ 6138, 6139, 9002, 9011

- primaries:  .092, .095, .098, .101, .104

- secondaries:  .131, .143, .149

- needle/seats:  .0935", .111"

- rods:  1998, 1999

10-112	TQ 6140, 6615, 9034

- primaries:  .092, .095, .098, .101, .104

- secondaries:  .131, .143, .149

- needle/seats:  .0935", .111"

- rods:  2000, 2001

10-113	TQ 6090

- primaries:  .092, .095, .098, .101, .104

- secondaries:  .131, .143, .149

- needle/seats:  .0935", .111"

- rods:  1995, 2005

10-114	TQ 6165, 6166, 9800

- primaries:  .092, .095, .098, .101, .104

- secondaries:  .131, .143, .149

- needle/seats:  .0935", .111"

- rods:  1996, 1997

10-115	TQ 6318, 6319, 6321, 6322, 6324, 6339, 3640, 6342, 6410, 6411, 6452,

6454, 6456, 6457, 6459, 6460, 6488, 6616, 9013, 9016, 9017, 9022,

9023, 9025

- primaries:  .092, .095, .098, .101, .104

- secondaries:  .131, .143, .149

- needle/seats:  .0935", .111"

- rods:  2002, ??

10-116	TQ 6320, 6341, 6453, 6455, 6461, 6496, 9004, 9008, 9009, 9012, 9014,

9015, 9024, 9046, 9049, 9050, 9051, 9052, 9053, 9054, 9063, 9072

- primaries:  .092, .095, .098, .101, .104

- secondaries:  .131, .143, .149

- needle/seats:  .0935", .111"

- rods:  2003, 2004

Later Kits, OEM and 9000 TQ:

- - - - - - - - - - - - - -

Kit #	Carb Type and Models Covered, with contents

------	---------------------------------------------

10-203	TQ All 1972 and later solid fuel type carburetors.

- primaries:  .092, .095, .098, .100, .101, .104

- secondaries:  .131, .137, .143, .149

- needle/seats:  .0935", .111"

- rods:  1996, 1997, 2002, 2003, 2004, 2005

Later Kits, AFB, AVS:

- - - - - - - - - - -

Kit #	Carb Type and Models Covered

------	----------------------------

10-201	AFB All 9000 series, all Competition Series, All OEM w/2 step rod.

10-202	AVS All, All OEM AFB w/3 step rod, high step primary jet.

Note: Special thanks to David Wright on the MML for providing copies of the

early documentation and typing the early kit contents and applications.

-------------------------------------------------------------------------------

References...

-------------

- Carter Service Manual # 3623A	"Thermo--QQuad"

- Carter Service Manual # 3630A "Basic FFaacts and Principles"

- Carter Carburetors catalog, # 3878, Apprril 1975

- Carter Floats catalog 1980

- Carter Carburetor95 catalog, # 3788, AAuugust 1977

- Carter Zip Kits & Carburetor Componenttss catalog, # 3880, 1991

- Carter Carburetor Emission Components  ccatalog, # 3881, 1989

- Carter Performance Products catalogs 119979, 1980, 1982, 1984

- Carter Strip Kit Specifications & Insttrructions, # 3270, 1-73, revised 11-75

- Carter Strip Kit label, # 3272, 7-75

- "Carter Carburetors", SA-Designs, Davee  Emanuel, 1983

- Chrysler Technical Guide, Thermo-Quad  CCarburetor, 1982, # 81-699-2009

- Chrysler Student Reference Book, Quadrraajet Carburetor, 1984, # 81-699-5003

- Chrysler factory service manuals, variioous, 1971-1984

- Chrysler parts catalogs, various, 19711--1984

- Chrysler Technical Service Bulletin #  114-01-72C, Dec. 15 1971.

- Direct Connection & Mopar Performance  ccatalogs

- Mopar Performance Engine Manual, # P522449010, 8th Edition

- Hot Rod magazine, June 1970

- Car Craft magazine, March 1972

- The Hot-Rodders Manual for Street & Sttrrip, '79-'80 & '81-'82 Annual (Hot Cars)

- Popular Hot Rodding, October 1973, Octtoober 1981

- High Performance Mopar magazine, Summeerr 1988.

- Mopar Action magazine, February 1991,  OOctober 1991.

- Mopar Muscle magazine, August/Septembeerr 1995.

- Chrysler Power magazine, January, Marcchh, May 1988.

- KEM Air & Fuel Management Systems Parttss, # AF-114, 1993

- KEM All Line Buyers' Guide, # BG-116,  11995

- Hygrade Carburetor Kit & Component Parrtts Guide, # FC-91, 1991

- Hygrade Carburetor & Fuel System Partss  Guide, # CM-88, 1988

- Hygrade Carburetion Component Guide, ##  FT-91, 1991

- Holley Replacement Carburetors catalogg,, # L-30033-4, Spring 1985

- Borg Warner Carburetor Kits catalog, ##  CK-79, Sept 1979

- Motorcraft Carburetor, Emission Parts  && Tune-up Kits, # F200W, 1985

- AC-Delco Carburetors & TBI, Carb Kits  && Parts, # 9A-100, 1986

- Tomco Remanufactured Carburetors & TBII  catalog, # D.I. 1991, 1991

- Kimco Carburetors, # 30-9, 3-1-83

- Kimco Carburetor Numerical Listing & OO..E.M. Section, # 30-9, 3-1-83

- Mopar Mailing List (MML) postings

- My memory (not so good) and my Thermo--QQuads (somewhat better).

-------------------------------------------------------------------------------

Acknowledgements...

-------------------

- Thanks to David Wright for early Stripp  Kit information.

- Thanks to Cam, Uncle Pete, Jim, and Daavvid for brass float information.

- Thanks to the MML in general.